# Example of a WMS server installation

## Installing the Internet Information Server (IIS) components to host the ASP.NET web service.

Figure 1

Figure 2

Figure 3

## Installing the WMS Web Service

The web service installation is located in the WMS\WMS WebService subfolder.

From the WMS\WMS WebService\wyUpdate3.7 folder, copy the client.wyc and wyUpdate.exe files (the version at the end of the folder - i.e., 3.7 - varies) first to the desktop of the target computer.

On the target machine, locate the "root" folder of the IIS server, which by default is C:\inetpub\wwwroot. Here we create a folder where the web service will be installed (e.g. WMS - the full path is C:\inetpub\wwwroot\wms\). Into this folder we copy the files we have previously copied to the desktop (Figure 4).

Figure 4

We run the wyUpdate file, which is configured to download the latest version from our servers.

We click Update, this will install the WebService

Figure 5

Configuration is still needed, but more on that when the database is set up. However, from the folder WMS\WMS WebService to the folder where the WMS was installed, copy the file "Web.config", where the settings are located, and which will be configured later according to the layout of the other components.

Once the basis for the web service has been set up, it must be edited in the IIS server to make it available for access. First, start IIS (Start -> IIS button). The screen as shown in Figure 6 appears (the folder we created in the IIS "root" folder is now visible, but not yet functional). We change the folder to an "application" as shown in Figure 7 (Convert to Application) and confirm the change (Figure 8)

Figure 6

Figure 7

Figure 8

The first test of the web service is done by right-clicking on the new site in IIS and testing it with "browse". See Figure 9.

Figure 9

The "service endpoints" of the web service (svc files - Figure 10) appear in the browser, which when clicked on they must show their interface (if OK, the content will be displayed with a content similar to Figure 11).

Figure 10

Figure 11

IMPORTANT

The web service is now working, but the permissions on the folder where it is installed are still missing. On this folder (in our case c:\inetpub\root\wms) we need to add the user "IUSR" with full control rights, otherwise certain web service commands do not work (PATCH commands). See Figure 12.

Figure 12

## Installing an MSSQL database

## Installing MS SQL Server Express (if SQL SERVER is already installed skip this)

Download the SQL Express installer from this link: https://www.microsoft.com/en-us/sql-server/sql-server-downloads

Figure 13

Figure 14

Figure 15

Figure 16

Figure 17

Figure 18

Figure 19

Select default instance and pick the name of the instance (for example WMS) shown in Figure 20.

Figure 20

Set the startup type of SQL Server Browser to Automatic (Figure 21). Before you click next switch to the "Collation" tab and select "Slovenian_CI_AS" (Figure 22).

Figure 21

Figure 22

On the Database Engine Configuration step select Mixed Mode under Authentication Mode and pick a password for the sa (system administrator) account (Figure 23).

Figure 23

## Install SQL Server Management Studio (skip this step is SSMS is already installed)

Download and install SSMS from this link: https://learn.microsoft.com/en-us/sql/ssms/download-sql-server-management-studio-ssms

## Add a user to the installed SQL Server

If we don’t already have a user for our SQL Server, we will open SSMS and login using the sa account we created during the SQL Server installation.

Figure 24

In the Object Explorer we go under Security  Login, right click on Login and select New Login…

Figure 25

We select SQL Server authentication pick our login name and password and we uncheck Enforce password expiration (Figure 26). Before we click OK on the left of the window we select the Server Roles and check the roles as shown in Figure 27.

Figure 26

Figure 27

## Creating a routing database (MasterRoutingDB)

The data backend of the LEOSS WMS application consists of two databases, a master database, and a routing database, which routes the user to the master database (via the routing database, we can thus have several master databases).

In the folder "WMS\WMS Snapshot MasterRoutingDB" there is the latest snapshot of the MasterRoutingDB database. Copy the file from this folder to the target server.

In Microsoft SQL Server Management studio we first create a new database called MasterRoutingDB (Figure 28).

Figure 28

Once the database is created, import the snapshot of the database as shown in Figure 29.

And click “Next” to the end.

Figure 29

Then enter the user in the RoutingTable table in the MasterRoutingDB database.

Run the SQL command below:

INSERT INTO RoutingTable (Username, Password, ConnString, RegisterDate)

VALUES (

'username',

'password',

'Server=Server;Database=WMS;', GETDATE()

)

## Creating a master database

In a similar way to how we created the routing database, we create the master database. From the folder "WMS\WMS Snapshot DB", select the most recent snapshot of the database and copy it to the target server.

In Microsoft SQL Management studio we create a new database with any name we want, but usually we stick to the rule that the production database is in WMS and the test database is in WMS_TEST (Figure 28).

Following the same procedure as for the routing database, import a snapshot of the master database (Figure 29).

## 4. Connecting the WMS Web Service to the database.

When installing the WMS Web Service, we copied the Web.config file to the base folder (by default this is c:\inetpub\wwwroot\WMS\web.config).

To edit, open the file in any txt file editor (Notepad will be OK).

From the configuration parameters, we find a node named CloudEntryPoint, where we enter the data to connect to the routing database (MasterRoutingDB). An example of a connection is shown in Figure

30. In the connectionString we set the “Server” value to the IP to the computer hosting the database, “Uid”  and “Pwd” is the user we created for the database.

Figure 30

## Installing the WMS Manager

The latest version of the WMS Manager application is located in the "WMS\WMS Manager" folder. Always select the latest version and copy it to the target server. Run the application and click "Next" to the end.

Figure 31

The first time you start the application, you need to set up the connection to the WMS Web Service. This is done by selecting Settings from the Login screen and entering the relevant information. An example is shown in Figure 32.

Figure 32

## Installing the WMS Komunikator and connector

The WMS Komunikator provides a standard interface to the LEOSS WMS system and is the same regardless of the ERP to which it is connected. The connector on the other hand takes care of converting the data from the ERP into a format suitable for the Komunikator and is therefore linked to the ERP.

At the moment there are three connectors, namely the connector for the connection to the Pantheon ERP, the SAOP ERP and the connector used when there is no ERP.

Pre-prepared installations are located in folders:

"WMS\WMS Komunikator\_INSTALL_Saop"

"WMS\WMS Komunikator\_INSTALL_Pantheon" and

"WMS\WMS Komunikator\_INSTALL_WithoutConector"

- Depending on the ERP to which we want to connect, copy the connector to the target server from the appropriate folder in our download the content.

- Inside the installation folder there is a folder called "LEOSS", which is copied to the folder "C:\Program Files (x86)" (The full folder is therefore "C:\Program Files (x86)\LEOSS\").

- Navigate to the folder "C:\Program Files (x86)\LEOSS\LEOSS WMS Komunikator" with explorer. In this folder we find the production components of the communicator and the connector for the specific ERP. First, we need to run the file wyUpdate.exe, which initiates the updates.

- In this folder, we look for the file "servis.cfg", which we need to open with a text editor (notepad) and insert the appropriate values. An example is shown in Figure 33.

- In the base folder (e.g. WMS\WMS Komunikator\_INSTALL_Saop\LEOSS), look for the file CommandInstallationService.txt. This contains the CMD command for registering the production and test communicator. This file is opened again with a text editor.

- Run the "Command line" command with Administrative rights and copy the contents of the text file CommandToReplaceService.txt to the command line. This will register the communicator for the selected ERP on the target system.

- After running the service installation command from CMD, we need to edit the services.txt file, where we enter the name of the service (the name is the parameter immediately after the sc create command. In the case of the wmskomunikator example file). Similar to the WebService configuration under ODBCConnString we set Server as the ip of the SQL Server and Database is the name of the Database (e.g. WMS). Under DatabaseUID is the Username of the database user and under DatabasePWD is its password.

- On the target system, we run the "Services" snap-in, where we can see all the services running on the system. Find the services in the list (Figure 34) and start them (this is only necessary the first time, after which the system will take care of starting itself if it restarts).

Figure 33

Figure 34

## ERP Connection Configuration

To configure the connection to the ERP we open WMS Manager, go to the System settings tab and then the System settings button (Figure 35).

Figure 35

## Connecting to an ERP through a database (e.g. PANTHEON)

If we are connecting to an ERP through a database, we fill the settings under Accounting software connection (ERP).

The stock compare table is always _LEOSS_StockCompare and order import status is 2, if we’re using Pantheon.

Figure 36

## Connecting to an ERP with a web API (e.g. SAOP)

If we’re connecting to an ERP with a web API, we fill the settings under Accounting software connection (ERP) with API.

Figure 37

When we fill out the settings to connect to an ERP we can check if it works by opening the Transfer log  window and clicking the Refresh button.

If the connection is successful, we will see messages that documents were successfully imported. Otherwise, errors will appear.

Figure 38